Pi Pico Installation and setup C/C++ Sdk

Global Installation steps

- Install the below tools, And while Installing add their path to environment variables

- CMake - https://cmake.org/download/

- Git -

- ARM GCC Toolchain - https://developer.arm.com/downloads/-/arm-gnu-toolchain-downloads
arm-gnu-toolchain-14.3.rel1-mingw-w64-i686-arm-none-eabi.exe

- Git

- Python

- Mingw-W64 - ( instead of build tools for visual studio 2019) - 
https://www.youtube.com/watch?v=oC69vlWofJQ
https://code.visualstudio.com/docs/cpp/config-mingw

  - In MYSYS2 shell run
> pacman -S --needed base-devel mingw-w64-ucrt-x86_64-toolchain

  - Edit the PATH variable (user env variable)and add:
C:\msys64\ucrt64\bin

  - Verify installation – open cmd and run
> gcc –version 
> g++ --version
> mingw32-make --version

- Cloning pico-sdk from github: https://github.com/raspberrypi/pico-sdk.git
> git clone -b master https://github.com/raspberrypi/pico-sdk.git
> cd pico-sdk 
> git submodule update –init

optional steps to download pico-examples

> cd ..

> git clone -b master https://github.com/raspberrypi/pico-examples.git

- Add pico-sdk path to environment variables
Under User variables add Path

Variable name: PICO_SDK_PATH

Variable value: E:\Embedded\pi_pico\pico-sdk

- Creating the first project blink led and building it using the sdk

- To build pico-examples

- Clone pico examples: > git clone -b master https://github.com/raspberrypi/pico-examples.git
> cd pico-examples
> mkdir build
> cd build

- For Mingw-W64 setup
> cmake -G "MinGW Makefiles" ..
> mingw32-make

- For visual studio build tools setup(not required because I’m proceeding with Mingw)
> cmake -G "NMake Makefiles" ..
> nmake

- Windows installation refer: https://shawnhymel.com/2096/how-to-set-up-raspberry-pi-pico-c-c-toolchain-on-windows-with-vs-code/

- Linux Installation refer: https://circuitdigest.com/microcontroller-projects/how-to-program-raspberry-pi-pico-using-c

Container based steps: Docker + Vs code editor

- Reference:
 https://github.com/AppacYazilim/PicoDevEnv/tree/main
https://github.com/lukstep/raspberry-pi-pico-docker-sdk/tree/main
linux docker setup: https://youtu.be/CYBmfYR3Ti8?si=9-tBZ97tlwHwuIYE

- DockerFile

Dockerfile
# Base image with build essentials

FROM ubuntu:latest

# Install necessary packages

RUN apt-get update && apt-get install -y \

cmake \

gcc-arm-none-eabi \

libnewlib-arm-none-eabi \

libstdc++-arm-none-eabi-newlib \

git \

python3 \

g++ \

&& rm -rf /var/lib/apt/lists/*

# Set a default workspace

WORKDIR /workspace

- Devcontainer.json

{

"name": "Pico Dev Container",

"build": {

"dockerfile": "Dockerfile",

"context": ".."

},

"workspaceFolder": "/workspace",

"postCreateCommand": "git clone https://github.com/raspberrypi/pico-sdk.git && cd pico-sdk && git submodule update --init",

"customizations": {

"vscode": {

"extensions": [

"ms-vscode.cpptools",

"ms-vscode.cmake-tools"

]

}

}

}

- Steps to build

- Open Your Workspace in VS Code

- Rebuild or Open the Dev Container

(OR)

How to rebuild or open the dev container in VS Code:

- Open VS Code.

- Open your project folder (that contains .devcontainer).

- Press Ctrl+Shift+P (or Cmd+Shift+P on Mac) to open the Command Palette.

- Type "Dev Containers: Rebuild Container" and select it.

  - This rebuilds the container from your Dockerfile and devcontainer.json (including reinstalling packages).

- Alternatively, if you haven't opened the folder in a container yet:

  - You will see an option to Reopen in Container; select it to launch the project inside the container.

- VS Code will then build or rebuild the container image, start the container, mount your workspace, and open a VS Code session connected to the container.

- Any terminals or commands you run inside VS Code now execute inside this isolated container.

Why do this?

- The container contains all your tools, libraries, and dependencies installed via the Dockerfile.

- Rebuilding applies any changes to your container environment.

- Opening inside the container gives you a consistent, reproducible dev environment across machines.

- Your code stays on your host machine but runs inside the container with full access.

- Make Sure Pico SDK is Cloned
Verify that pico-sdk exists in your workspace folder and that PICO_SDK_PATH is correctly set (it should be /workspace/pico-sdk).

- Edit or Add Your Project Source Code
Work on your C/C++ files and CMakeLists.txt inside your project folders (e.g., /workspace/001_blink/). Files ->add folder to workspace…

- Build Your Project

  - Open a terminal in VS Code (`Ctrl+``).

  - Navigate to your project directory (e.g., cd 001_blink).

> mkdir build 
> cd build

- Run cmake to configure the project
> cmake ..

- Build the project:
> make

6. Locate Build Artifacts

- After building, the output files (.uf2, .elf, .bin) will be in your project’s build folder.

- These files are available in both the container and host filesystem for easy use and flashing.

7. Sync Code and Artifacts

- Any files or changes you make inside /workspace (container) will appear in your project folder on the host machine.

- You can use version control, drag-and-drop, or VS Code commands to sync, move, or copy build artifacts as needed.

- Files ->add folder to workspace…

Optional: > export PICO_SDK_PATH=/workspace/pico-sdk/

export PICO_SDK_PATH=/workspaces/pico_docker_env/pico-sdk

Standard Project Structure

CMakeLists.txt

cmake_minimum_required(VERSION 3.13)

include(pico_sdk_import.cmake)

project(my_pico_project)

pico_sdk_init()

add_executable(my_pico_app

src/main.c

src/helper.c

)

target_include_directories(my_pico_app PRIVATE include)

target_link_libraries(my_pico_app pico_stdlib)

pico_add_extra_outputs(my_pico_app)

usage
# Set SDK path

export PICO_SDK_PATH=/path/to/pico-sdk

# Build project

mkdir build && cd build

cmake ..

make

Multicore

https://www.youtube.com/watch?v=aIFElaK14V4&list=PLEB5F4gTNK68IlRIJtcJ_2cW4dSdmreTw&index=7&t=311s

https://learnembeddedsystems.co.uk/basic-multicore-pico-example

I2C

    - Features

Each I2C controller is based on a configuration of the Synopsys DW_apb_i2c (v2.01) IP. The following features are supported:

- Master or Slave (Default to Master mode)

- Standard mode, Fast mode or Fast mode plus

- Default slave address 0x055

- Supports 10-bit addressing in Master mode

- 16-element transmit buffer

- 16-element receive buffer

- Can be driven from DMA

- Can generate interrupts

Check the sensor board schematics to check if the pullup resistors are included in the pcb board of the sensor itself

Pico SDK i2c most useful functions

- i2c_init

- i2c_write_blocking

- i2c_read_blocking

write operation -> i2c_write_blocking

read operation -> i2c_write_blocking and i2c_read_blocking(sends the address of specific register where we want to read and then reads from the register)

mpu6050 - I2C device found at address 0x68  !

bmp280 - I2C device found at address 0x76  !

FreeRTOS on the Raspberry Pi Pico (RP2040) - Step-by-Step FreeRTOS Setup

Step 1: Prerequisites

Ensure you have these installed:

On linux
> sudo apt install cmake gcc-arm-none-eabi build-essential git

On windows

- Pico SDK (already set up from your previous projects)

- CMake (3.13 or later)

- GCC ARM toolchain (arm-none-eabi-gcc)

- Git

Step 2: Download FreeRTOS Kernel

cd /pico_freertos_project

git clone https://github.com/FreeRTOS/FreeRTOS-Kernel.git

cd FreeRTOS-Kernel

git checkout main  #Or specific version like V10.5.1

Step 3: Complete Project Structure

Step 4: Create FreeRTOSConfig.h - Location: config/FreeRTOSConfig.h

#ifndef FREERTOS_CONFIG_H

#define FREERTOS_CONFIG_H

/*-----------------------------------------------------------

* Application specific definitions.

*

* These definitions should be adjusted for your particular hardware and

* application requirements.

*

* THESE PARAMETERS ARE DESCRIBED WITHIN THE 'CONFIGURATION' SECTION OF THE

* FreeRTOS API DOCUMENTATION AVAILABLE ON THE FreeRTOS.org WEB SITE.

*

* See http://www.freertos.org/a00110.html

*----------------------------------------------------------*/

/* Scheduler Related */

#define configUSE_PREEMPTION                    1

#define configUSE_TICKLESS_IDLE                 0

#define configUSE_IDLE_HOOK                     0

#define configUSE_TICK_HOOK                     0

#define configTICK_RATE_HZ                      ( ( TickType_t ) 1000 )

#define configMAX_PRIORITIES                    32

#define configMINIMAL_STACK_SIZE                ( configSTACK_DEPTH_TYPE ) 256

#define configUSE_16_BIT_TICKS                  0

#define configIDLE_SHOULD_YIELD                 1

/* Synchronization Related */

#define configUSE_MUTEXES                       1

#define configUSE_RECURSIVE_MUTEXES             1

#define configUSE_APPLICATION_TASK_TAG          0

#define configUSE_COUNTING_SEMAPHORES           1

#define configQUEUE_REGISTRY_SIZE               8

#define configUSE_QUEUE_SETS                    1

#define configUSE_TIME_SLICING                  1

#define configUSE_NEWLIB_REENTRANT              0

// todo need this for lwip FreeRTOS sys_arch to compile

#define configENABLE_BACKWARD_COMPATIBILITY     1

#define configNUM_THREAD_LOCAL_STORAGE_POINTERS 5

/* System */

#define configSTACK_DEPTH_TYPE                  uint32_t

#define configMESSAGE_BUFFER_LENGTH_TYPE        size_t

/* Memory allocation related definitions. */

#define configSUPPORT_STATIC_ALLOCATION         0

#define configSUPPORT_DYNAMIC_ALLOCATION        1

#define configTOTAL_HEAP_SIZE                   (128*1024)

#define configAPPLICATION_ALLOCATED_HEAP        0

/* Hook function related definitions. */

#define configCHECK_FOR_STACK_OVERFLOW          0

#define configUSE_MALLOC_FAILED_HOOK            0

#define configUSE_DAEMON_TASK_STARTUP_HOOK      0

/* Run time and task stats gathering related definitions. */

#define configGENERATE_RUN_TIME_STATS           0

#define configUSE_TRACE_FACILITY                1

#define configUSE_STATS_FORMATTING_FUNCTIONS    0

/* Co-routine related definitions. */

#define configUSE_CO_ROUTINES                   0

#define configMAX_CO_ROUTINE_PRIORITIES         1

/* Software timer related definitions. */

#define configUSE_TIMERS                        1

#define configTIMER_TASK_PRIORITY               ( configMAX_PRIORITIES - 1 )

#define configTIMER_QUEUE_LENGTH                10

#define configTIMER_TASK_STACK_DEPTH            1024

/* Interrupt nesting behaviour configuration. */

/*

#define configKERNEL_INTERRUPT_PRIORITY         [dependent of processor]

#define configMAX_SYSCALL_INTERRUPT_PRIORITY    [dependent on processor and application]

#define configMAX_API_CALL_INTERRUPT_PRIORITY   [dependent on processor and application]

*/

#if FREE_RTOS_KERNEL_SMP // set by the RP2040 SMP port of FreeRTOS

/* SMP port only */

#define configNUMBER_OF_CORES                   2

#define configTICK_CORE                         0

#define configRUN_MULTIPLE_PRIORITIES           1

#define configUSE_CORE_AFFINITY                 1

#define configUSE_PASSIVE_IDLE_HOOK             0

#endif

/* RP2040 specific */

#define configSUPPORT_PICO_SYNC_INTEROP         1

#define configSUPPORT_PICO_TIME_INTEROP         1

#include <assert.h>

/* Define to trap errors during development. */

#define configASSERT(x)                         assert(x)

/* Set the following definitions to 1 to include the API function, or zero

to exclude the API function. */

#define INCLUDE_vTaskPrioritySet                1

#define INCLUDE_uxTaskPriorityGet               1

#define INCLUDE_vTaskDelete                     1

#define INCLUDE_vTaskSuspend                    1

#define INCLUDE_vTaskDelayUntil                 1

#define INCLUDE_vTaskDelay                      1

#define INCLUDE_xTaskGetSchedulerState          1

#define INCLUDE_xTaskGetCurrentTaskHandle       1

#define INCLUDE_uxTaskGetStackHighWaterMark     1

#define INCLUDE_xTaskGetIdleTaskHandle          1

#define INCLUDE_eTaskGetState                   1

#define INCLUDE_xTimerPendFunctionCall          1

#define INCLUDE_xTaskAbortDelay                 1

#define INCLUDE_xTaskGetHandle                  1

#define INCLUDE_xTaskResumeFromISR              1

#define INCLUDE_xQueueGetMutexHolder            1

/* A header file that defines trace macro can be included here. */

#endif /* FREERTOS_CONFIG_H */

Step 5: Create Top-Level CMakeLists.txt

Step 7: Build the Project

cd pico_freertos_project

mkdir build

cd build

cmake ..

make -j4

Zephyr RTOS on the Raspberry Pi Pico (RP2040) - Step-by-Step Zephyr RTOS Setup

For Zephyr rtos setup refer: https://docs.zephyrproject.org/latest/develop/getting_started/index.html

Step 1: Install dependencies,

> winget install Kitware.CMake Ninja-build.Ninja oss-winget.gperf Python.Python.3.12 Git.Git oss-winget.dtc wget 7zip.7zip

Step 2: Install west and Zephyr

- Create a workspace directory: >mkdir ~/pico_zephyrproject && cd ~/pico_zephyrproject

- Create Python venv and activate:

- >python3 -m venv .venv

- > .\zephyr_venv\Scripts\Activate.ps1

- Install west (Zephyr meta-tool) via pip:

- > python.exe -m pip install --upgrade pip

- >pip install west​​

- Fetch Zephyr source:

- >west init -m https://github.com/zephyrproject-rtos/zephyr.git zephyrproject or >west init zephyrproject

- >cd zephyrproject

- >west update​​

- Export Zephyr CMake package and install Python deps:

- >west zephyr-export

- >pip install -r zephyr/scripts/requirements.txt

Step 3: Install Zephyr SDK

- Download Zephyr SDK minimal bundlefrom the official site and extract it to e.g. E:Embedde/pico/zephyr-sdk.​ https://github.com/zephyrproject-rtos/sdk-ng/releases/tag/v0.17.4
download zephyr-sdk minimal bundle because its allows us to select the required toolchain alone instead of installing all the toolchains

- Extract the downloaded file zephyr-sdk-0.17.4_windows-x86_64_minimal.7z and move the extracted folder to C:\

- Run  this in cmd >C:\ zephyr-sdk-0.17.4\setup.cmd 
then select arm-zephyr-eabi-gcc alone by typing Y for installation

- To verify arm toolchain exists 
C:\zephyr-sdk-0.17.4\arm-zephyr-eabi\bin>arm-zephyr-eabi-gcc --version

arm-zephyr-eabi-gcc (Zephyr SDK 0.17.4) 12.2.0

Copyright (C) 2022 Free Software Foundation, Inc.

This is free software; see the source for copying conditions.  There is NO

warranty; not even for MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.

- Tell Zephyr where sdk is (set environment variables)
> setx ZEPHYR_SDK_INSTALL_DIR “C:\zephyr-sdk-0.17.4”

| Aspect | Minimal Bundle | Full Bundle |
| --- | --- | --- |
| Toolchains included | None—empty folder initially | All architectures pre-installed |
| Disk space | 100-200 MB after adding one toolchain (e.g., ARM) | 5-6 GB with all toolchains |
| Installation process | Download minimal → runsetup.sh(Linux/macOS) orsetup.cmd(Windows) → select specific toolchains interactively | Extract archive → toolchains ready immediately |
| Use case | Single-target projects (e.g., only RP2040/ARM Cortex-M) or disk-constrained environments | Multi-architecture projects or exploration across different boards |
| Host tools | Includes OpenOCD, QEMU, debuggers (same as full) | Same host tools included |

Run the SDK setup script: (In linux)

- >cd ~/zephyr-sdk

- >./setup.sh​

- Set environment variable (add to your shell rc file):

- In windows: > set ZEPHYR_SDK_INSTALL_DIR= E:Embedde/pico/zephyr-sdk

- In linux: > export ZEPHYR_SDK_INSTALL_DIR=~/zephyr-sdk

Step 4: Test raspberry pi pico build

>cd F:\Embedded\pico_zephyr_project\zephyrproject\zephyr

>west build –b rpi_pico samples\basic\blinky

Step 5: flash

Put pico in BOOTSEL mode, then

>west flash

Or drag and drop

>build\zephyr\zephyr.uf2

Important Reference Links:

https://docs.zephyrproject.org/latest/develop/getting_started/index.html
	https://www.hackster.io/cdwilson/zephyr-rtos-on-raspberry-pi-pico-2-part-1-cf39f0

https://github.com/raspberrypi/pico-zephyr

Digikey - Zephyr Projects tutorial

Supported Boards : https://docs.zephyrproject.org/latest/boards/index.html#

Zephyr workspace: https://docs.zephyrproject.org/latest/develop/west/workspaces.html

Zephyr docker image: https://github.com/ShawnHymel/introduction-to-zephyr

https://github.com/maksimdrachov/zephyr-rtos-tutorial.git

Simple Explanation: T1, T2, T3 (Zephyr Workspace Types)

- T1: star topology, zephyr is the manifest repository

- T2: star topology, a Zephyr application is the manifest repository

- T3: forest topology, freestanding manifest repository

Think of a Zephyr workspace like a folder that holds many Git projects (Zephyr, libraries, apps).
The difference between T1, T2, T3 is who controls the workspace using west.yml.

T1 – Star Topology (Zephyr-centric) - “Zephyr is the boss”

- The Zephyr repository contains the west.yml file.

- All modules and projects are defined by Zephyr.

- Best if you are:

  - Contributing to Zephyr

  - Following Zephyr’s official structure

workspace/

├─ zephyr/   ← has west.yml

├─ modules/

└─ tools/

- T2 – Star Topology (Application-centric) - “My app is the boss”

- Your application repo has the west.yml.

- Zephyr is just another dependency.

- Best if you are:

  - Building a single product or firmware

  - Controlling Zephyr version yourself

workspace/

├─ my_app/   ← has west.yml

├─ zephyr/

└─ modules/

- T3 – Forest Topology - “Many independent projects, one workspace”

- A separate manifest repo controls everything.

- Multiple apps + Zephyr + modules coexist.

- Best if you are:

  - Managing multiple projects

  - Working in a company or lab environment

workspace/

├─ manifest/   ← has west.yml

├─ zephyr/

├─ app1/

├─ app2/

└─ modules/

- ⭐ Quick Comparison Table

| Type | Who owns west.yml | Best for |
| --- | --- | --- |
| T1 | Zephyr repo | Zephyr development |
| T2 | Your app repo | Single product |
| T3 | Separate manifest repo | Multiple projects |

- How to Create a Zephyr Workspace Using T3 – Forest Topology

Update your system packages:

- sudo apt update

- sudo apt upgrade

Install required host dependencies:

- sudo apt install --no-install-recommends git cmake ninja-build gperf \ccache dfu-util device-tree-compiler wget python3-dev python3-venv python3-tk \xz-utils file make gcc gcc-multilib g++-multilib libsdl2-dev libmagic1

  - On ARM64 systems, skip gcc-multilib and g++-multilib.

  - Verify versions:

- cmake --version  # Should be >= 3.20.5

- python3 --version  # Should be >= 3.10

- dtc --version  # Should be >= 1.4.6

Create a workspace directory (e.g., ~/zephyrproject):

- mkdir ~/zephyrproject

- cd ~/zephyrproject

Set up a Python virtual environment (recommended to isolate dependencies):

- python3 -m venv .venv

- source .venv/bin/activate
pip install --upgrade pip

  - Your prompt will show (.venv). Reactivate it in future sessions with source .venv/bin/activate.

Install the west tool (Zephyr's workspace manager):

- pip install west

### Set Up the Zephyr Workspace (Using Forest Topology)

zephyr_workspace/

├─ .west/

├─ manifest/

│   └─ west.yml

├─ zephyr/

├─ app1/

├─ app2/

└─ modules/

Create a Minimal west Manifest (only clone what's needed for Pico)

mkdir manifest

cd manifest

create a file west.yml in manifest repo

manifest:

remotes:

- name: zephyrproject-rtos

url-base: https://github.com/zephyrproject-rtos

projects:

- name: zephyr

remote: zephyrproject-rtos

repo-path: zephyr

revision: main

path: zephyr

import:

name-allowlist:

- hal_rpi_pico     # Required for RP2040 support

- cmsis            # Cortex-M core support

- cmsis_6

self:

path: manifest

Initialize the workspace:

- cd ~/zephyrproject
west init -l manifest/

  - This sets up the manifest and prepares the forest structure.

Update the repositories (downloads Zephyr and all modules):

- west update

Export Zephyr for CMake integration:

- west zephyr-export

Install additional Python dependencies:

- west packages pip-install
or
pip install -r zephyr/scripts/requirements.txt

  - This installs packages required by Zephyr modules.

Install the Zephyr SDK (toolchain):

- cd zephyr  # Navigate to the Zephyr repo inside the workspace

- west sdk install

  - This installs cross-compilers, debuggers, and simulators for all architectures. Use west sdk install --help for options like specific architectures.

OR Install only the ARM toolchain (minimal SDK)

- cd ~/zephyrproject

west sdk install -t arm-zephyr-eabi

- This downloads and extracts just the arm-zephyr-eabi toolchain (~600 MB compressed, ~1.2–1.5 GB extracted).

- It places it in ~/zephyrproject-pico/zephyr-sdk-<version> (or similar /home/ezhil/zephyr-sdk-0.17.4).

- No other architectures (x86, riscv, xtensa, etc.) are installed.

If the command above fails or you want even more control, manually download & setup:

cd ~

wget https://github.com/zephyrproject-rtos/sdk-ng/releases/download/v0.17.0/zephyr-sdk-0.17.0_linux-x86_64_arm-zephyr-eabi.tar.xz   # check latest version!

tar xvf zephyr-sdk-*.tar.xz

cd zephyr-sdk-*

./setup.sh

Then tell west where it is

export ZEPHYR_SDK_INSTALL_DIR=~/zephyr-sdk-0.17.0   # adjust version

west config manifest.project-filter -- +hal_rpi_pico +cmsis

But west sdk install -t arm-zephyr-eabi is usually cleaner

### Create the apps Directory with a Blink LED Example

Zephyr allows custom applications in a workspace directory (e.g., apps) alongside the core repos. We'll copy the standard "blinky" sample (a basic LED blink example) and place it there.

- Create the apps directory in your workspace:

- cd ~/zephyrproject  # Back to workspace root

- mkdir apps

- Copy the blinky example into apps:

- cp -r zephyr/samples/basic/blinky apps/blink_led

  - This creates apps/blink_led with the necessary files: CMakeLists.txt, prj.conf, and src/main.c (which toggles an LED using Zephyr's GPIO API).

- Verify or customize the example (optional):

  - Open apps/blink_led/src/main.c in an editor. It should look like this (simplified):

- #include <zephyr/device.h>

- #include <zephyr/drivers/gpio.h>

- #include <zephyr/kernel.h>

- #define LED_NODE DT_ALIAS(led0)

- static const struct gpio_dt_spec led = GPIO_DT_SPEC_GET(LED_NODE, gpios);

- int main(void) {

- gpio_pin_configure_dt(&led, GPIO_OUTPUT_ACTIVE);

- while (1) {

- gpio_pin_toggle_dt(&led);

- k_msleep(1000);  // Blink every second

- }

- return 0;

- }

  - Ensure your target board has an LED defined in its devicetree (e.g., led0 alias). No changes needed for most boards.

### Build and Run the Blink LED Example

- Build the application:

- west build -p auto -b <your-board-name> -s apps/blink_led

- west build -p auto -b rpi_pico apps/blink_led

  - Replace <your-board-name> with your hardware (e.g., nrf52dk_nrf52832 for Nordic nRF52 DK). List supported boards with west boards.

  - The -p auto enables pristine build (cleans previous builds).

  - Output goes to build/ in the workspace.

- Flash to your board:

- west flash

  - Connect your board via USB. This uploads the firmware and runs it (LED should blink).

- Debug or run in simulator (optional, if no hardware):

- west build -p auto -b qemu_cortex_m3 -s apps/blink_led

- west debug

  - Uses QEMU for simulation.

Minicom for serial output - step-by-step guide to view Raspberry Pi Pico UART (serial) output over USB using minicom on Linux.

## 1. Prepare the Pico firmware

Make sure your Pico code is actually sending output to UART over USB.

### Common cases

- USB CDC (stdio over USB)
Using the Pico SDK with:

- stdio_init_all();

- and in CMakeLists.txt:

- pico_enable_stdio_usb(your_target 1)

- pico_enable_stdio_uart(your_target 0)

- UART via GPIO (UART0/UART1)
This will NOT work over USB alone. You’d need a USB-to-UART adapter.
If you’re using only the Pico’s USB cable, you must use USB CDC.

⚠️ If you’re unsure: USB cable only → use USB CDC

## 2. Connect the Pico to your computer

- Plug the Pico into your Linux machine via USB

- Power it normally (no BOOTSEL needed once firmware is flashed)

## 3. Find the Pico’s serial device

>ls /dev /ttyACM* /dev/ttyUSB*

Typical result:

/dev/ttyACM0

If multiple devices exist:

dmesg | grep tty

## 4. Install minicom (if not installed)

>sudo apt update

>sudo apt install minicom

## 5. Configure minicom

Run setup:

>sudo minicom -s

### Inside the setup menu:

- Select Serial port setup

- Set:

  - Serial Device → /dev/ttyACM0

  - Bps/Par/Bits → 115200 8N1
(baud rate doesn’t matter for USB CDC, but set it anyway)

  - Hardware Flow Control → No

  - Software Flow Control → No

- Press Enter to exit serial setup

- Select Save setup as dfl

- Exit minicom

## 6. Run minicom

>minicom

You should now see Pico output such as:

Hello, Pico!

## 7. If you see nothing

Try these checks:

### A. Permissions

Add yourself to the dialout group:

sudo usermod -a -G dialout $USER

Then log out and back in.

### B. Reset the Pico

Unplug and replug the Pico
(or press the Pico’s RESET button)

### C. Test with screen

Quick test without config:

screen /dev/ttyACM0 115200

Exit with:

Ctrl+A → K → Y

## 8. Common mistakes

❌ Using UART GPIO without USB-UART adapter
❌ Expecting output before stdio_init_all()
❌ Wrong /dev/ttyACM* device
❌ Flow control left enabled in minicom

## 9. Optional: Auto-connect shortcut

>sudo minicom -D /dev/ttyACM0 -b 115200